Target center is assumed to be 3.2 cm downstream relative to the NC center.

Target thickness is 15 cm, and the inner-radius is 11.75 cm. Please check these numbers again.

At least, the target center must be consistent what the analysis data assumed. Otherwise it will introduce bias

to the corrected momentum.   Very unfortunately, the modification of experimental

apparatus position requires the compilation.

Sample data and the resulting files (including pdf-cenverted top-drawer) are

produced by simulation without assuming K^-pnn (=\overline{K^0}nnn) formation.

expected input file 1

analized data in sub-directory ./proton-data/data.dat

formatted 11 real numbers in one line for each event

read(luni, *, err = 10) k_trg_cross, NC_proton_point, proton_TOF, p_p_input, dE_thin, dE_thick, vCA_vp

k_trg_cross(3) : kaon and pion vertex point in (cm) unit

NC_proton_point(3) : NC hit point in (cm) unit

proton_TOF : proton TOF (nsec) unit

p_p_input : proton initial guess (MeV/c) unit  - can be a dummy number

dE_thin : TC_thin normalized pulse hight (MeVee/6mm)  - can be a dummy number

dE_thick : TC_thin normalized pulse hight (MeVee/30mm)  - can be a dummy number

vCA_vp  : v_{CA}*v_n (mm) unit  - can be a dummy number

expected input file 2

PID data in sorce-directory ./boundary.dat

sample is given (never mind (forget about it) if you do not care for topdrower output file)

-25. -5. 	: region to be selected for v_{CA}*v_n in (mm) unit

7		: number of vertices of a polygon in "dE_thick(MeVee/30mm)" and "dE_thin(MeVee/6mm)" plane

2. 0.5	: position of 1st vertex

10. 0.5	: position of 2nd vertex

12.57143 0.62857		.........

11.18002 2.01998		.........

10. 1.9			.........

7. 3.				.........

2.0 2.0	: position of last vertex

Onput file 1

corrected data in sub-directory ./proton-data/output_data.dat

formatted 11 real numbers in one line for each event - same format but replaced "p_p_input"

p_p_output : energy loss corrected proton momentum (MeV/c) unit

Onput file 2

topdrower source file in sub-directory ./proton-data/data.top